FLOTILLA-LEVEL FIELD SOP • COMMUNICATIONS

VHF/HF Radio Facility Inspection

A combined documentation, technical, and net-readiness checklist for examining a member's VHF and/or HF radio station offered as a U.S. Coast Guard Auxiliary operational facility.

Field aid, not the official record. This checklist is a working tool for flotilla examiners. It does not replace the current Auxiliary Manual (COMDTINST M16790.1), your District's Facility program SOPs, or the official Facility Examination form — confirm current 7004 form, required signatures, and standards with your FSO-CM / SO-CM / DSO-CM or DIRAUX before filing results.

| Owner/Operator, Member Number | Call Signs (VHF/HF) | TeleCommunications Operator (TCO) Qualification Date |
| --- | --- | --- |
| Facility Type | Frequency Ranges | Flotilla/Division |
| Examiner’s Name, Member Number | Prior Inspection Date | Inspection Date |

A. Documentation & Administrative Review

|  | Operator's Auxiliary membership and required Communications specialty training are current |
| --- | --- |
|  | Facility offer / agreement for use is on file and matches equipment being inspected |
|  | Equipment list (make, model, serial) matches what is physically installed |
|  | Operator holds a current TCO certification |
|  | Prior year's inspection report on file, deficiencies (if any) corrected |
|  | Address of Radio Facility and Latitude/Longitude agree of Form 7004 |
|  | Ensure that the Form 7004 and AUXDATA II agree |

B. VHF Equipment & Installation

|  | Transceiver powers up cleanly; make/model/serial recorded |
| --- | --- |
|  | Transmit power within spec on high and low settings ( ~25 W high /1 W low, fixed or mobile marine VHF) |
|  | Ch. 16 and required working/weather channels programmed and receive clearly |
|  | Antenna, coax, and connectors in good condition; connections weatherproofed |
|  | SWR measured at feedpoint and within spec < 2:1 acceptable, < 1.5:1_preferred |
|  | Antenna mount secure; coax routed with drip loop, no chafe points |

C.  HF Equipment & Installation

|  | Transceiver powers up cleanly; make/model/serial recorded |
| --- | --- |
|  | Frequency coverage includes assigned Auxiliary/CG and/or net working frequencies |
|  | Transmit power matches the equipment rating |
|  | Antenna tuner present and achieves match across bands in use |
|  | SWR acceptable on each band used for net operations |
|  | Antenna type/height suits mission — NVIS for regional net coverage vs. higher/ directional for long-haul |
|  | Ground / counterpoise system adequate for antenna type |

D.  Power Systems

|  | Primary power connection secure and properly fused |
| --- | --- |
|  | Backup power source present and sized to run the station through an outage Battery / generator / solar - confirm runtime against unit's expected net duration |
|  | Battery voltage checked under transmit load |
|  | Operator can demonstrate switch-over to backup power without assistance |

E. Grounding, Lightning & Safety

|  | Station ground rod / bond to structural ground present and intact |
| --- | --- |
|  | Coax surge suppressor and a means to disconnect/ground antenna feedlines when not in use |
|  | Antenna placement keeps personnel clear of the RF exposure zone at rated power |
|  | Cabling secured, no trip hazards; outdoor connections weatherproofed |

F.  Operational & Net Readiness

|  | Voice check completed with NCS or another qualified station on the VHF working channel |
| --- | --- |
|  | Voice check completed with NCS or another qualified station on the HF working frequency. Note propagation/time-of-day if signal was marginal |
|  | Signal report obtained and logged e.g. readability/strength "5x5" or RST |
|  | Alternate/backup frequency known and tested |
|  | Operator demonstrates net log procedure (paper or digital) and can record a check-in correctly |
|  | Operator can describe emergency/priority traffic handling procedure |

G.  Examiner Findings & Certification

| PASS / CONDITIONAL / FAIL | Describe any items marked unsatisfactory and the date by which they must be corrected. |
| --- | --- |
| Examiner’s Signature |  |
|  | Date |
| Owner/Operator’s Signature | Next Inspection Due Date |

Suggested cadence: re-examine at least every three years, and after any change of equipment, antenna, or installation location. Retain a copy with the flotilla's facility records and forward per your District's process.